JOHOR PORT MARINE

1. Table indicating numbers of Pilot and Tug to be engaged for a particular vessel

- Channeling

- Ship to ship

- Special assignment

- Deep draft

- Towing Dead Vessel

| Type of assignment | Vessel LOA | Nos. of Pilot | Tugs Requirement |  |
| --- | --- | --- | --- | --- |
| Channeling | All sizes of vessel | 1 | Comply to Sembawang Harbor transit rules | Comply to JPB / Sembawang Harbor transit rules |
| Berthing / Unberthing | Less than 125m | 1 | 1 tugs exceptional to BT 4 , BT 7 and BT 8 = 2 tugs | Comply to JPB Safety Standard |
| Berthing / Unberthing | More than 125m | 1 | - tugs Vessel with Bow thruster is allowed to use 1 tug depending on the Hp of the bow thruster. | Comply to JPB Safety Standard |
| Ship to ship | Less than 240m | 1 | 2tugs | No restriction |
|  | 240m to 270m | 1 | 2-3tugs | No restriction |
|  | Above 270m | 2 | 3-4tugs | Recommended for daylight berthing only |
|  | VLCC-ULCC(Laden) | 2 | 4-5tugs | Recommended for daylight berthing only |
| Special assignment (Oil rigs /drill rigs) | Above 80m | 2 (1tow+1craft) | Comply to Sembawang Harbor transit rules | Comply to JPB / Sembawang Harbor transit rules |
| *Deep draft | All vessel (draft of 12m and above) | 2 | 1 escort tug | Comply to JPB / Sembawang Harbor transit rules |
| Towing dead vessel | From 80m -100m | 1 | 2 tugs | Comply to JPB / Sembawang Harbor transit rules |
|  | Above 100m | 2 | 2-3-4tugs Depend on the Load of Cargo onboard | Comply to JPB / Sembawang Harbor transit rules |
| *Anchoring | VLCC-ULCC(Laden) | 2 | 2 to 4 Tugs As per Request Minimum 2 | At the discretion of Marine HoD |